Jetson Re-flash procedure

- Install https://docs.nvidia.com/sdk-manager/install-with-sdkm-jetson/index.html  on ubuntu machine

- Connect JetsonTx2 to host machine with the original USB-microUSB cable

- Open SDK Manager on host machine (Target Hardware should detect inserted device). Press “Continue To Step2”

- Click on “I accept the terms and conditions of the license agreement”. Click “Continue To Step 03”. You will be prompted to create /home/USER/nvidia/nvidia_sdk and /home/ USER /Downloads/nvidia/sdkm_downloads folders. Click “Create” for both. Enter root password when prompted. Wait for downloads to finish.

- Insert current username and password and click “Flash”

At this moment, the JetsonTx2 restarts in Recover mode and starts flashing the device:

When the Flash ends, you will be able to complete the “System configuration wizard” on Jetson.

- Complete the “System configuration wizard” on Jetson with default settings (English, etc.). The JetsonTx2 will restart automatically.

- Login and connect JetsonTx2 to a network (lan/wifi)

- On host machine, insert the username and password from your freshly installed Jetson. Click Install

- Wait for install process to finish. Click “Finish and exit”

- Move on Jetson and start install Tensorflow, Pytorch, Torchvision

  - TENSORFLOW

- sudo apt-get update - sudo apt-get install libhdf5-serial-dev hdf5-tools libhdf5-dev zlib1g-dev zip libjpeg8-dev liblapack-dev libblas-dev gfortran - sudo apt-get install python3-pip - sudo pip3 install -U pip - sudo pip3 install -U pip testresources setuptools==49.6.0 - sudo pip3 install cython - sudo pip3 install -U numpy==1.19.4 future==0.18.2 mock==3.0.5 h5py==2.10.0 keras_preprocessing==1.1.1 keras_applications==1.0.8 gast==0.2.2 futures protobuf pybind11 - sudo pip3 install --pre --extra-index-url https://developer.download.nvidia.com/compute/redist/jp/v45 tensorflow

  - PYTORCH

- sudo apt-get install libopenblas-base libopenmpi-dev - download pip wheel from jetson zoo. For example Pytorch v1.8.0: https://nvidia.box.com/shared/static/p57jwntv436lfrd78inwl7iml6p13fzh.whl - cd Downloads - sudo pip3 install torch-1.8.0-cp36-cp36m-linux_aarch64.whl

  - TORCHVISION

- sudo apt-get install libjpeg-dev zlib1g-dev - git clone https://github.com/pytorch/vision torchvision - cd torchvision - sudo python3 setup.py install

  - OTHER DEPENDENCIES

- sudo pip3 install psutil